PEM 841.741

Superficie: 410

€/m2: 1.819 €/m2

Can Carol es un símbolo para los vecinos del barrio de Vallcarca. Se trata de una antigua masía del S.XIX vinculada a su pasado rural. El edificio se encontraba prácticamente derruido; únicamente quedaban en pie las fachadas y algunos muros principales. Tras largas reivindicaciones, en 2015 fue adquirido por el ayuntamiento con el objetivo de convertirlo en un equipamiento sociocultural autogestionado por las entidades del barrio.

Los muros que todavía se conservaban delimitaban tres áreas que coincidían con las superficies principales del programa: una sala polivalente, un paquete de aulas/taller y una pequeña cantina; al mismo tiempo nos revelaban las fases constructivas del edificio.

La intervención aborda la transformación de la ruina en equipamiento preservando su memoria histórica pero sin caer en la restitución. La rehabilitación parte de la estructura y resuelve a través de un único material, la madera, la estabilidad, el aislamiento y el acabado interior de los nuevos espacios, en un gesto que aúna sostenibilidad, economía y expresividad.

Los muros exteriores han sido rehabilitados mediante técnicas y materiales tradicionales. Por el interior, una nueva estructura autoportante de CLT estabiliza las antiguas fachadas y resuelve el cerramiento y acabado interior en una misma hoja. Únicamente en el “espacio atrio”, en contacto con la medianera posterior, los muros de mampostería se muestran vistos haciendo patente la condición interior / exterior de este nuevo espacio bioclimatizado que se utiliza para mejorar el confort y reducir el consumo energético.

Can Carol is a symbol for the residents of the Vallcarca neighbourhood. Built in the 19th century, it is an old farmhouse linked to its rural origins. The building was practically in ruins; only the facades and some main walls remained standing. After long protest movements, in 2015 it was acquired by the city council with the aim of turning it into a socio-cultural facility, self-managed by neighborhood entities.

The walls that were still preserved delimited three areas that coincided with the main surfaces of the program: a multipurpose room, a package of classrooms/workshops, and a small canteen; at the same time they revealed to us the various construction phases of the building.

The intervention deals with the transformation of the ruin into a facility, preserving its historical memory but without falling into restitution. The rehabilitation starts from the structure and solves through a single material, wood, stability, insulation and the interior finish of the new spaces, in a gesture that combines sustainability, economy and expressiveness.

The exterior walls have been rehabilitated using traditional techniques and materials. On the inside, a new CLT self-supporting structure stabilizes the old facades and resolves the enclosure and interior finish on the same sheet. Only in the "atrium space", in contact with the rear dividing wall, the masonry walls remain exposed, making clear the interior / exterior condition of this new bioclimatized space that is used to improve comfort and reduce energy consumption.